## Confocal image processing protocol

## 02/2026 - Inês Marques

## 1. Data preparation and preprocessing

## 1.1. Raw data and acquisition

- Acquire whole‑fish confocal stacks with three channels: nuclear H2B‑GCaMP6s (panneuronal), DsRed reporter (gad1b), and in situ HCR v3.0 (Alexa 647).

- Use a ZEISS LSM 980 with Airyscan 2, 40×/1.2 NA water‑immersion objective, Multiplex 4Y mode, zoom 1.1×, voxel size ≈0.05 × 0.05 × 0.25 µm, 10 tiles with ~10% overlap, reduced stage velocity/acceleration (~20% of default), frame‑sequential acquisition from 640 → 561 → 488 nm, GCaMP laser power ramped from ~0.8% at the top to ~1.5% at the bottom; a full 3‑channel whole‑fish dataset requires ~8.5 h under these conditions.

Manuel: The laser intensity refers to equivalent power in confocal mode, not to the actual power. The resolution is actually 0.065 x 0.065 x 0.25 (there is some kind of bug in the software, sometimes it appears as 0.05 and others as 0.065. But the former makes the acquisitions too long)

- Define three imaging channels:

  - Channel 1 (GCaMP6s): 488 nm excitation, 500–550 nm emission.

  - Channel 2 (DsRed): 561 nm excitation, 570–620 nm emission.

  - Channel 3 (Alexa 647 in situ): 640 nm excitation, 650–700 nm emission.

- For each fish, create a folder with subfolders for raw stacks, preprocessed stacks, and 3D blocks for segmentation

## 1.2. Airyscan Reconstruction

Transfer czi file to MERLIN Computer( it is possible to do this also in confocal computer)

Then access Zen, and click module Airyscan Processing or Airyscan Joint Deconvolution. I used Airyscan Processing because it is a more simple deconvolution that takes less time and when imagens have more than 80Gb is impossible to do Joint deconvolution.

In Airyscan Processing choose 3D and then we can choose sparse, standard and dense, depending on the type of image and fluorescece we have. For my files I choose Standard.

## in ZEN Airyscan Processing module, input the czi file and click apply. Depending on the size of the image it will take while.

## 1.3. Tiling and stitching

Manuel: I used BigStitcher instead

- Stitch the Airyscan tiles in ZEN (“Stitching” module) to obtain a single 3D volume per fish.

- Export stitched stacks as 16‑bit, three‑channel TIFFs with voxel calibration embedded in the metadata.

## 1.4. Background removal and denoising

- On the bleach‑corrected nuclear channel:

  - Downscale in XY if needed to ≈0.1 µm/pixel using bilinear interpolation, keeping the original Z sampling(FIJI).

  - Subtract background with the rolling‑ball algorithm in FIJI, using a radius approximately equal to or slightly larger than the largest nuclear radius (~50 pixels).

  - On the nuclear GCaMP channel, correct depth‑dependent attenuation with the bleachcorrection.ipynb script (e.g. histogram matching per Z‑slice while preserving relative contrast).

  - Apply 3D median filtering to remove salt‑and‑pepper noise while preserving edges.

  - Apply CLAHE slice‑wise (blocksize ≈127, 256 bins, max slope ≈3) to enhance local contrast and recover dim nuclei.

  - Optionally apply a mild anisotropic 3D Gaussian blur (more smoothing in Z than XY) to reduce z‑elongation and make nuclei more isotropic.

- Apply the same resampling, cropping and, if needed, smoothing to the DsRed and in situ channels so that all channels remain spatially aligned with the preprocessed nuclear channel.

## 2. Model development and benchmarking (single reference volume)

This section is done once on a representative fish to choose the segmentation model; only after this selection is made is the chosen model applied to all fish.

## 2.1. Ground‑truth annotations

- Select one representative hindbrain volume (preprocessed nuclear channel).

- Manually annotate all nuclei in 3D in napari (or equivalent) to obtain a dense 3D ground truth label volume( I did in one volume with 600x700x 140 slices.

- For Cellpose training: extract slices in three orthogonal views (XY, XZ, YZ) and annotate ~19 slices per view (~57 slices total, >2000 nuclei).

- For StarDist3D and SwinCell: use the 3D label volume directly.

- For Cellpose (2D, orthogonal views):

  - Use Partiremdiferentesperspetivas.ipynb to generate XY/ZX/ZY views and 2D crops.

  - Create three folders (one per view), each with ≈20 augmented slices for human‑in‑the‑loop training in the Cellpose GUI.

## 2.2. Data augmentation

For cellpose data aumentations were done with: need to find the script

- For StarDist3D and SwinCell:

  - Use dataumentations3Dlatest.py to apply 3D augmentations (rotations, flips, elastic deformations, intensity scaling, etc.) to the volume and labels.an

## 2.3. Training individual models

- Train StarDist3D with train_stardist3D3dez.py, using the 3D GT and the optimised patch size, learning rate and loss weights.

- Train SwinCell with trainswincell2dezcomval.py, using Dice loss on the mask and MSE on the flow field, with the hyperparameters that gave stable convergence.

- Fine‑tune Cellpose cyto3:

  - Start from the generic “cyto3” model.

  - Use the Cellpose GUI in human‑in‑the‑loop mode on the three orthogonal view folders.

  - Save the fine‑tuned model (“cyto3 ft”) for 3D inference.

## 2.4. Model benchmarking and selection

- On the same annotated 3D reference volume, run inference with:

  - Fine‑tuned Cellpose cyto3 and other cellulose models(via Cellpose CLI, command below).

  - StarDist3D (predict_stardist3d3dez.py).

  - SwinCell(inferswincell3dez.py )

- Compute, with comparisonmodels.py:

  - Voxelwise Dice, IoU, Precision, Recall.

  - Instance‑wise average precision (AP) at multiple IoU thresholds (AP0.5, AP0.75, AP0.5–0.95).

  - Volume distributions (model vs ground truth) to assess size bias.

- Choose the final segmentation model according to:

  - Highest voxelwise Dice/IoU.

  - Highest AP across IoU thresholds 0.5–0.95.

  - Acceptable runtime and GPU memory requirements for whole‑fish volumes.

- In this pipeline, the fine‑tuned Cellpose cyto3 model achieved the best balance between accuracy (e.g. Dice ≈0.88, IoU ≈0.78 on the reference volume) and practicality, and was selected as the model to apply to all fish.

## 3. Full‑volume segmentation with the chosen model

From this point onward, all fish are processed with the single selected segmentation model (here, fine‑tuned Cellpose).

## 3.1. Block‑wise segmentation of whole fish

- Because each stitched volume is ~11 GB, perform segmentation in overlapping Z‑blocks: e.g. 0–100, 80–200, 160–260, 240–end slices, with ~20‑slice overlap.

- Keep the full XY field of view for every block.

- For each block, run Cellpose on the nuclear channel (or a three‑channel stack if using multi‑channel inference), with a command similar to:

powershell

cellpose `

--image_path "D:\User data\...\Downscaledwholefish3_3channelshalfleft_apenasnuclei_ZattenuationCorrected_Cellpose_0a100clahe.tif" `

--pretrained_model "D:\User data\...\models cellpose\hindbrain_cyto3_finetune" `

--restore_type denoise_cyto3 `

--use_gpu `

--do_3D `

--anisotropy 2.5 `

--flow_threshold 0.4 `

--cellprob_threshold -5 `

--stitch_threshold 0.4 `

--niter 600 `

--min_size 2000 `

--save_tif `

--save_outlines `

--save_png `

--no_npy `

--output_name fish5_180a300_denoise3Dflow `

--verbose `

--flow3D_smooth 3

- Use 3D mode, anisotropy ≈2.5, cellprob_threshold = −5 and flow3D_smooth = 3 to maximise sensitivity and obtain smooth flows.

## 3.2. Block merging and de‑duplication

- For each fish, merge the four block‑level label volumes into one full‑volume label map with a custom merging script:

  - In overlapping regions, match labels using IoU or centroid distance and keep a single label per cell.

  - Resolve conflicts by preferring the instance with higher confidence (or larger overlap with neighbours) if such a measure is available.

## 3.3. Post‑processing of segmented nuclei

Post-processing is actually done before the block merging in the code - Manuel

- Run postprocessingfill.py on the merged segmentation:

  - Fill small holes in nuclear masks.

  - Smooth jagged boundaries to obtain more rounded nuclei.

- Optionally, filter instances by volume (e.g. remove objects below a minimum volume threshold or above a maximum expected nuclear size).

## 3.4. Overlap classification bookkeeping

This overalp_full.py is also done in the block stage, and its not just ‘a file mapping each nucleus …’ it is the classification for the in-situ and dsred. How do i merge this data afterwards? This contradicts the point 4… I can’t run the thing on the whole volume - Manuel

- Run overlap_full.py to produce a file mapping each nucleus to the blocks where it was detected.

- Use this mapping in downstream analysis to ensure each cell is counted only once, even if it originally appeared in overlapping blocks.

## 4. Co‑expression classification (DsRed and in situ)

## 4.1. Manual point annotations

- For each fish, select at least three representative Z‑slices spanning superficial, mid‑depth and deep regions.

- Open the corresponding TIFFs in FIJI, including:

  - Nuclear segmentation masks.

  - DsRed channel.

  - In situ (Alexa 647) channel.

- Overlay masks and fluorescence channels with partial transparency.

- Using the Multipoint tool, click the centre of each visually positive cell (DsRed or in situ, depending on the label being annotated).

- Save all points to the ROI Manager and export as a table; XM and YM columns store the image coordinates and serve as ground‑truth labels.

## 4.2. DsRed feature selection

- Run the DsRed analysis script to:

  - Load nuclear masks and DsRed channel.

  - For every annotated slice, compute per‑cell metrics (mean intensity, median, max, integrated density, fraction of voxels above a percentile).

  - Quantify how well each metric separates manually labelled positive and negative cells (e.g. ROC/AUC, F1 after thresholding).

- Use the metric that performs best; in this dataset, the fraction of bright voxels above ~P85–P90 separated positives and negatives best and was chosen as the DsRed score.

## 4.3. In situ shell optimisation

- Run 3jan_shellchoose.py to optimise shell geometry and intensity threshold for in situ classification:

  - Define perinuclear shells via inner and outer radii in pixels (inner radius can be negative to extend into the nucleus; outer radius is positive, covering perinuclear cytoplasm).

  - For each (inner, outer) pair and each percentile P (e.g. P60–P95):

    - Compute per‑cell fraction of voxels above P within the shell.

    - Apply Otsu threshold to this fraction to classify cells as in situ+ / in situ−.

    - Compute F1, Precision, Recall versus manual labels.

- Inspect the resulting metric curves and heatmaps to find a robust setting; here, P85–P90, and particularly P90, gave the most stable F1 across shells.

- One high‑performing shell used an inner radius of −5 px and an outer radius of +3 px, capturing a small intranuclear region plus a compact perinuclear shell.

## 4.4. Final classification

- Run the main classification script with:

  - Inputs: full 3D DsRed TIFF, in situ TIFF, and the nuclear mask volume.

  - Parameters:

    - DsRed: bright‑pixel fraction at P85–P90 with an automatically determined (e.g. Otsu) threshold.

    - In situ: shell geometry (inner −5 px, outer +3 px) and percentile P90 with Otsu threshold on the bright‑pixel fraction.

- For each nucleus:

  - Compute the DsRed score and in situ score.

  - Apply thresholds to label the cell as DsRed+/in situ+, DsRed+/in situ−, DsRed−/in situ+, or DsRed−/in situ−.

  - Save a table containing nucleus ID, centroid coordinates, volume, and binary labels for DsRed and in situ.

## 4.5. Permutation‑based validation

- Run dsredpermutation.py to assess whether classifier performance is above chance:

  - Shuffle class labels within each slice while preserving the number of positives and negatives.

  - Recalculate F1 for each shuffle (e.g. 5000 iterations) to build a null distribution.

  - Compare the observed mean F1 to the null distribution and compute an empirical p‑value.

- In this pipeline, the observed F1 fell far above the null distribution (empirical p ≈0.0002), supporting that classification reflects meaningful signal rather than chance.

## 5. Application to all fish

- For every fish:

  - Apply the preprocessing in Section 1.

  - Segment with the single chosen model from Section 2 (e.g. fine‑tuned Cellpose).

  - Merge and post‑process labels, and compute overlap bookkeeping (Section 3).

  - Apply the fixed DsRed and in situ classification parameters determined once in Section 4 (do not re‑optimise per fish).

This ordering ensures that model selection and parameter optimisation are done once on a reference dataset, and then consistently applied to all fish before computing overlaps and classification statistics.